# =============================================================================

# SE-aware Bandwidth Allocation + Alternating Bandwidth-Power Optimization

# for Multi-UAV 6G Networks  —  complete simulation (Google Colab ready)

#

# Base paper: M. A. Alnakhli, E. M. Mohamed, M. M. Fouda, "Bandwidth Allocation

# and Power Control Optimization for Multi-UAVs Enabled 6G Network",

# IEEE Access, vol. 12, pp. 67405-67415, 2024.

#

# HOW TO RUN IN COLAB

#   1. Open https://colab.research.google.com -> New notebook

#   2. Paste this whole file into one cell and press Run (Shift+Enter)

#      (quick test first: set TRIALS = 20; final paper run: TRIALS = 200)

#   3. When finished, a file "results.zip" downloads automatically

#      (CSV tables + PNG figures for the paper).

# =============================================================================

TRIALS       = 200          # Monte Carlo trials per configuration (use 20 to test)

UAV_COUNTS   = [3, 4, 5, 6, 7, 8]

LARGE_COUNTS = [9, 10]      # scalability study

DENSITIES    = [0.5, 0.8, 1.0]

BETA         = 0.8          # QoS: each link must keep >= BETA of its stage-1 rate

MAX_ALT_IT   = 20           # max iterations of the alternating stage

SEED         = 2026

OUT          = "results"

import os, time, itertools, zipfile, warnings

import numpy as np, pandas as pd, networkx as nx

import matplotlib; matplotlib.use("Agg"); import matplotlib.pyplot as plt

from scipy.optimize import linprog, minimize

from scipy.stats import wilcoxon

warnings.filterwarnings("ignore")

os.makedirs(OUT, exist_ok=True)

# ---------------------------------------------------------------- parameters

BTOT, PMAX = 10e6, 1.0                      # Hz, W            (base Table 1)

PL0, D_V   = 46.4, 50.0                     # dB, m            (base Table 1)

RHO0 = 10 ** ((-147 + 10*np.log10(BTOT) + 10 - 30) / 10)   # W (base Table 1)

CHANNELS = {"LoS (base)":            dict(alpha=2.03, shadow=0.0),

"LoS + 4 dB shadowing":  dict(alpha=2.03, shadow=4.0),

"Mixed LoS/NLoS":        dict(alpha=2.03, shadow=6.0, nlos_alpha=3.0, p_los=0.7)}

# ---------------------------------------------------------------- scenario

def make_scenario(M, density, rng, alpha=2.03, shadow=0.0, nlos_alpha=None, p_los=1.0):

"""Random multi-UAV network exactly as in Sec. V of the base paper."""

dH = np.zeros((M, M)); iu = np.triu_indices(M, 1)

dH[iu] = rng.uniform(1, 100, len(iu[0])); dH = dH + dH.T

d = np.sqrt(dH**2 + D_V**2); np.fill_diagonal(d, 1.0)

a = np.full((M, M), alpha)

if nlos_alpha is not None:

los = rng.uniform(size=(M, M)) < p_los; los = np.triu(los, 1); los = los | los.T

a = np.where(los, alpha, nlos_alpha)

pl = PL0 + 10*a*np.log10(d)

if shadow > 0:

sh = rng.normal(0, shadow, (M, M)); sh = np.triu(sh, 1); pl = pl + sh + sh.T

g = 10 ** (-pl / 10)

pairs = list(itertools.combinations(range(M), 2))

n_e = max(1, int(round(density * len(pairs))))

links = [pairs[i] for i in sorted(rng.choice(len(pairs), n_e, replace=False))]

G = nx.Graph(); G.add_nodes_from(range(M)); G.add_edges_from(links)

active = np.array([G.degree(v) > 0 for v in range(M)])

L = nx.line_graph(G)                                   # link graph (Fig. 3)

mis = [set(c) for c in nx.find_cliques(nx.complement(L))] if len(links) > 1 else [set(L.nodes)]

pos = {e: k for k, e in enumerate(links)}

S = np.zeros((len(mis), len(links)))                   # MIS matrix, Eq. (9)

for q, c in enumerate(mis):

for e in c: S[q, pos[tuple(sorted(e))]] = 1

return dict(M=M, g=g, links=links, S=S, active=active)

def dir_se(P, sc):

"""Per-direction spectral efficiency log2(1+SINR), Eq. (4)-(5); shape (K,2)."""

g, act = sc["g"], sc["active"]; out = np.zeros((len(sc["links"]), 2))

for k, (i, j) in enumerate(sc["links"]):

for c, (t, r) in enumerate(((i, j), (j, i))):

m = act.copy(); m[t] = m[r] = False

out[k, c] = np.log2(1 + P[t]*g[t, r] / ((P[m]*g[m, r]).sum() + RHO0))

return out

def total_rate(sc, B, P):            # bit/s

return float((B[:, None] * dir_se(P, sc)).sum())

# ---------------------------------------------------------------- stage 1

def fairness_tau(S, gk):

"""Largest tau<=1 for which constraint (11.f) B_k >= tau*Btot*g_k is feasible."""

Q, K = S.shape

r = linprog(np.r_[np.zeros(Q), -1], A_ub=np.hstack([-S.T, (BTOT*gk)[:, None]]),

b_ub=np.zeros(K), A_eq=np.r_[np.ones(Q), 0][None], b_eq=[BTOT],

bounds=[(0, None)]*Q + [(0, 1)], method="highs")

return min(1.0, r.x[-1]) * (1 - 1e-9)

def bw_base(sc, R):

"""Base paper, problem (11): max sum_k B_k  (spectral efficiency NOT in objective).

Solved exactly (it is an LP); SQP of the base paper converges to the same point."""

S = sc["S"]; Q = S.shape[0]; gk = R / R.sum(); tau = fairness_tau(S, gk)

r = linprog(-S.sum(1), A_ub=-S.T, b_ub=-tau*BTOT*gk, A_eq=np.ones((1, Q)),

b_eq=[BTOT], bounds=[(0, None)]*Q, method="highs")

return r.x

def bw_se_aware(sc, R):

"""PROPOSED stage 1: max sum_k B_k R_k  (original objective (6)) under the same

constraints (11.a)-(11.f). Exact LP -> global optimum."""

S = sc["S"]; Q = S.shape[0]; gk = R / R.sum(); tau = fairness_tau(S, gk)

r = linprog(-(S @ R), A_ub=-S.T, b_ub=-tau*BTOT*gk, A_eq=np.ones((1, Q)),

b_eq=[BTOT], bounds=[(0, None)]*Q, method="highs")

return r.x

def bw_equal(sc, R):  return np.full(sc["S"].shape[0], BTOT / sc["S"].shape[0])

def bw_sqp_random(sc, R, rng):

"""SLSQP on (11) from a random start (Algorithm 1 of the base paper)."""

S = sc["S"]; Q = S.shape[0]; gk = R / R.sum(); tau = fairness_tau(S, gk)

res = minimize(lambda x: -S.sum(1) @ x, rng.dirichlet(np.ones(Q)), jac=lambda x: -S.sum(1),

method="SLSQP", bounds=[(0, 1)]*Q,

constraints=[{"type": "eq", "fun": lambda x: x.sum() - 1},

{"type": "ineq", "fun": lambda x: S.T @ x - tau*gk}],

options=dict(maxiter=300))

return np.clip(res.x, 0, None) * BTOT

# ---------------------------------------------------------------- stage 2

def min_power(sc, B, dem):

"""min sum Pt s.t. per-direction rate >= dem, 0<=Pt<=Pmax  (problem (19)).

With phi fixed this is linear in Pt; solved exactly as an LP (identical to the

M-matrix / Sherman-Morrison solution when each UAV has one link)."""

g, act, M = sc["g"], sc["active"], sc["M"]; rows = []

for k, (i, j) in enumerate(sc["links"]):

for c, (t, r) in enumerate(((i, j), (j, i))):

if dem[k, c] <= 0: continue

if B[k] <= 1.0: return None

phi = 2 ** min(dem[k, c] / B[k], 60) - 1

row = np.zeros(M); m = act.copy(); m[t] = m[r] = False

row[t] = -g[t, r] / RHO0; row[m] = phi * g[m, r] / RHO0

rows.append(row / phi)

if not rows: return np.zeros(M)

r = linprog(np.ones(M), A_ub=np.array(rows), b_ub=-np.ones(len(rows)),

bounds=[(0, PMAX) if a else (0, 0) for a in act], method="highs")

return r.x if r.status == 0 else None

def bw_headroom(sc, P, dem):

"""PROPOSED alternating step: for fixed powers, re-allocate MIS bandwidth to

maximise the common QoS headroom t:  B_kd * SE_kd(P) >= t * dem_kd  (LP)."""

S = sc["S"]; Q, K = S.shape; se = dir_se(P, sc); rows = []

for k in range(K):

for c in range(2):

if dem[k, c] > 0:

row = np.zeros(Q + 1); row[:Q] = -S[:, k]*se[k, c]/dem[k, c]; row[Q] = 1

rows.append(row)

r = linprog(np.r_[np.zeros(Q), -1], A_ub=np.array(rows), b_ub=np.zeros(len(rows)),

A_eq=np.r_[np.ones(Q), 0][None], b_eq=[BTOT],

bounds=[(0, None)]*(Q + 1), method="highs")

return r.x[:Q]

def alternating(sc, X, P, dem):

"""Block-coordinate descent; total power is non-increasing at every step

(the previous P stays feasible for the new bandwidths), so it converges."""

hist = [P.sum()]

for _ in range(MAX_ALT_IT):

Xn = bw_headroom(sc, P, dem); Pn = min_power(sc, sc["S"].T @ Xn, dem)

if Pn is None or Pn.sum() >= P.sum() * (1 - 1e-4): break

X, P = Xn, Pn; hist.append(P.sum())

return X, P, hist

# ---------------------------------------------------------------- one trial

def run_trial(sc, rng):

out = {}

P0 = np.where(sc["active"], PMAX, 0.0)

se0 = dir_se(P0, sc); R = se0.sum(1)                      # R_k, Eq. (7)

S = sc["S"]

def pipeline(name, X, alt=False, pc=True):

t0 = time.perf_counter()

B = S.T @ X

dem = BETA * B[:, None] * se0

P = min_power(sc, B, dem) if pc else P0

it = 0

if P is None: P = P0

if alt and pc:

X, P, h = alternating(sc, X, P, dem); B = S.T @ X; it = len(h) - 1

el = (time.perf_counter() - t0) * 1e3

rl = (B[:, None] * dir_se(P, sc)).sum(1)               # per-link rate

dr = rl.sum(); pw = P.sum()

out[name] = dict(DR=dr/1e6, EE=dr/pw/1e9 if pw > 0 else np.nan, Ptot=pw,

Jain=rl.sum()**2/(len(rl)*(rl**2).sum()), MinRate=rl.min()/1e6,

alt_iters=it, t_ms=el)

t0 = time.perf_counter(); Xb = bw_base(sc, R); tb = (time.perf_counter()-t0)*1e3

t0 = time.perf_counter(); Xs = bw_se_aware(sc, R); ts = (time.perf_counter()-t0)*1e3

Xq = bw_sqp_random(sc, R, rng)

pipeline("Base SQP (Alnakhli 2024)", Xb)

pipeline("Base SQP-WoPC", Xb, pc=False)

pipeline("Base SLSQP random start", Xq)

pipeline("Equal MIS bandwidth", bw_equal(sc, R))

pipeline("Proposed: SE-aware only", Xs)

pipeline("Proposed: alternating only", Xb, alt=True)

pipeline("Proposed (SE-aware + alternating)", Xs, alt=True)

pipeline("Proposed-WoPC", Xs, pc=False)

# Random Selection: random bandwidth and random power (base paper)

Br = rng.uniform(0, 1, len(R)); Br *= BTOT / Br.sum(); Pr = rng.uniform(0, PMAX, sc["M"]) * sc["active"]

rl = (Br[:, None] * dir_se(Pr, sc)).sum(1); dr = rl.sum()

out["Random Selection"] = dict(DR=dr/1e6, EE=dr/Pr.sum()/1e9, Ptot=Pr.sum(),

Jain=rl.sum()**2/(len(rl)*(rl**2).sum()),

MinRate=rl.min()/1e6, alt_iters=0, t_ms=0)

out["Base SQP (Alnakhli 2024)"]["t_stage1_ms"] = tb

out["Proposed (SE-aware + alternating)"]["t_stage1_ms"] = ts

return out

def run_config(M, dens, trials, chan="LoS (base)", tag="main"):

rows = []

for t in range(trials):

rng = np.random.default_rng(SEED + 100000*M + int(1000*dens) + t)

sc = make_scenario(M, dens, rng, **CHANNELS[chan])

for meth, v in run_trial(sc, rng).items():

rows.append(dict(exp=tag, channel=chan, N=M, density=dens, trial=t, method=meth, **v))

return rows

# ---------------------------------------------------------------- statistics

PROP = "Proposed (SE-aware + alternating)"

def summarise(df):

res = []

for (exp, ch, N, d), g in df.groupby(["exp", "channel", "N", "density"]):

ref = g[g.method == PROP].sort_values("trial")

for m, gm in g.groupby("method"):

gm = gm.sort_values("trial"); row = dict(exp=exp, channel=ch, N=N, density=d, method=m)

for met in ("DR", "EE", "Jain", "MinRate", "Ptot", "t_ms", "alt_iters"):

x = gm[met].dropna().values

row[f"{met}_mean"] = x.mean(); row[f"{met}_std"] = x.std(ddof=1)

row[f"{met}_ci95"] = 1.96*x.std(ddof=1)/np.sqrt(len(x)); row[f"{met}_median"] = np.median(x)

if m != PROP:

for met in ("DR", "EE"):

a, b = ref[met].values, gm[met].values

row[f"gain_{met}_%"] = 100*(a.mean()/b.mean() - 1)

try: row[f"p_{met}"] = wilcoxon(a, b, alternative="greater").pvalue

except ValueError: row[f"p_{met}"] = 1.0

res.append(row)

s = pd.DataFrame(res)

for met in ("DR", "EE"):                       # Holm correction within each config

for key, g in s.groupby(["exp", "channel", "N", "density"]):

idx = g[f"p_{met}"].dropna().sort_values().index; m = len(idx); run = 0

for r_, i in enumerate(idx):

run = max(run, min(1, (m - r_) * s.at[i, f"p_{met}"])); s.at[i, f"p_{met}_holm"] = run

return s

# ---------------------------------------------------------------- run everything

all_rows = []

T0 = time.time()

for d in DENSITIES:

for M in UAV_COUNTS:

all_rows += run_config(M, d, TRIALS); print(f"main  density={d}  N={M}  done  ({time.time()-T0:.0f}s)")

for M in LARGE_COUNTS:

all_rows += run_config(M, 0.5, max(20, TRIALS//4), tag="scale"); print(f"scale N={M} done")

for ch in list(CHANNELS)[1:]:

for M in (4, 6, 8):

all_rows += run_config(M, 0.8, TRIALS, chan=ch, tag="channel"); print(f"channel {ch} N={M} done")

raw = pd.DataFrame(all_rows); raw.to_csv(f"{OUT}/raw_all_trials.csv", index=False)

summ = summarise(raw); summ.to_csv(f"{OUT}/summary_stats.csv", index=False)

# beta sweep (QoS level vs EE trade-off)

beta_rows = []

for b in (0.5, 0.6, 0.7, 0.8, 0.9):

BETA = b

for r in run_config(6, 0.8, max(20, TRIALS//2), tag=f"beta={b}"):

if r["method"] in (PROP, "Base SQP (Alnakhli 2024)"): r["beta"] = b; beta_rows.append(r)

BETA = 0.8

bdf = pd.DataFrame(beta_rows)

bdf.groupby(["beta", "method"])[["DR", "EE", "Jain"]].agg(["mean", "std"]).to_csv(f"{OUT}/beta_sweep.csv")

# convergence curve of the alternating stage (N=5, density 0.8)

conv = []

for t in range(30):

rng = np.random.default_rng(777 + t); sc = make_scenario(5, 0.8, rng)

P0 = np.where(sc["active"], PMAX, 0.0); se0 = dir_se(P0, sc); R = se0.sum(1)

X = bw_se_aware(sc, R); B = sc["S"].T @ X; dem = BETA * B[:, None] * se0

P = min_power(sc, B, dem)

if P is None: continue

_, _, h = alternating(sc, X, P, dem); conv.append(np.array(h) / h[0])

# ---------------------------------------------------------------- figures

main = summ[summ.exp == "main"]

show = ["Proposed (SE-aware + alternating)", "Base SQP (Alnakhli 2024)",

"Equal MIS bandwidth", "Random Selection"]

style = {show[0]: ("o", "-"), show[1]: ("s", "--"), show[2]: ("^", ":"), show[3]: ("x", "-.")}

for met, lab, log in (("DR", "Sum data rate (Mbps)", False), ("EE", "Energy efficiency (Gbit/J)", True)):

for d in DENSITIES:

fig, ax = plt.subplots(figsize=(5.2, 3.8))

for m in show:

g = main[(main.density == d) & (main.method == m)].sort_values("N")

mk, ls = style[m]

ax.errorbar(g.N, g[f"{met}_mean"], yerr=g[f"{met}_ci95"], marker=mk, ls=ls,

capsize=3, ms=5, label=m)

ax.set_xlabel("Number of UAVs"); ax.set_ylabel(lab); ax.grid(alpha=.3)

if log: ax.set_yscale("log")

ax.set_title(f"Network density = {d}  (mean ± 95% CI, {TRIALS} trials)", fontsize=9)

ax.legend(fontsize=7); fig.tight_layout(); fig.savefig(f"{OUT}/fig_{met}_density_{d}.png", dpi=300)

plt.close(fig)

# ablation bar chart (N=6, density 0.8)

abl = ["Base SQP (Alnakhli 2024)", "Proposed: alternating only", "Proposed: SE-aware only",

"Proposed (SE-aware + alternating)"]

g = main[(main.N == 6) & (main.density == 0.8)].set_index("method").loc[abl]

fig, ax = plt.subplots(1, 2, figsize=(8, 3.4))

for a, met, lab in zip(ax, ("DR", "EE"), ("Sum data rate (Mbps)", "EE (Gbit/J)")):

a.bar(range(4), g[f"{met}_mean"], yerr=g[f"{met}_ci95"], capsize=3, color=["#888", "#9ab", "#69c", "#247"])

a.set_xticks(range(4)); a.set_xticklabels(["Base", "+Alt.", "+SE-aware", "Full"], fontsize=8)

a.set_ylabel(lab); a.grid(axis="y", alpha=.3)

fig.suptitle("Ablation (N = 6, density = 0.8)", fontsize=9); fig.tight_layout()

fig.savefig(f"{OUT}/fig_ablation.png", dpi=300); plt.close(fig)

# convergence

fig, ax = plt.subplots(figsize=(5, 3.5))

L = max(len(c) for c in conv)

C = np.array([np.r_[c, np.full(L - len(c), c[-1])] for c in conv])

for c in C: ax.plot(c, color="#ccc", lw=.8)

ax.plot(C.mean(0), "o-", color="#247", label="mean")

ax.set_xlabel("Alternating iteration"); ax.set_ylabel("Total power / initial")

ax.set_title("Monotone convergence of the alternating stage", fontsize=9)

ax.grid(alpha=.3); ax.legend(); fig.tight_layout(); fig.savefig(f"{OUT}/fig_convergence.png", dpi=300)

plt.close(fig)

# ---------------------------------------------------------------- print paper tables

pd.set_option("display.width", 200); pd.set_option("display.max_columns", 20)

cols = ["N", "density", "method", "DR_mean", "DR_ci95", "EE_mean", "EE_ci95", "Jain_mean",

"gain_DR_%", "gain_EE_%", "p_DR_holm", "p_EE_holm"]

key = ["Proposed (SE-aware + alternating)", "Base SQP (Alnakhli 2024)", "Equal MIS bandwidth", "Random Selection"]

print("\n=== TABLE A: main results (mean ± 95% CI); gains/p-values are Proposed vs. method ===")

print(main[main.method.isin(key)][cols].round(4).to_string(index=False))

print("\n=== TABLE B: ablation ===")

print(main[main.method.isin(abl + ["Proposed-WoPC", "Base SQP-WoPC", "Base SLSQP random start"])

& main.N.isin([4, 6, 8]) & (main.density == 0.8)][cols].round(4).to_string(index=False))

print("\n=== TABLE C: scalability and channel robustness ===")

print(summ[summ.exp.isin(["scale", "channel"]) & summ.method.isin(key[:2])]

[["exp", "channel"] + cols].round(4).to_string(index=False))

print("\n=== TABLE D: runtime (ms per network) ===")

print(summ[summ.method.isin(key[:2]) & (summ.exp != "channel")].groupby(["N", "method"])["t_ms_mean"].mean().round(2).to_string())

print("\n=== TABLE E: QoS factor beta sweep (N=6, density 0.8) ===")

print(bdf.groupby(["beta", "method"])[["DR", "EE", "Jain"]].mean().round(4).to_string())

print(f"\nTotal time {time.time()-T0:.0f} s")

with zipfile.ZipFile("results.zip", "w") as z:

for f in os.listdir(OUT): z.write(os.path.join(OUT, f))

try:

from google.colab import files; files.download("results.zip")

except Exception:

print("results.zip written (download it from the Colab file panel if it did not start)")

Outputs:

main  density=0.5  N=3  done  (18s)

main  density=0.5  N=4  done  (39s)

main  density=0.5  N=5  done  (48s)

main  density=0.5  N=6  done  (60s)

main  density=0.5  N=7  done  (72s)

main  density=0.5  N=8  done  (93s)

main  density=0.8  N=3  done  (100s)

main  density=0.8  N=4  done  (110s)

main  density=0.8  N=5  done  (123s)

main  density=0.8  N=6  done  (134s)

main  density=0.8  N=7  done  (161s)

main  density=0.8  N=8  done  (186s)

main  density=1.0  N=3  done  (198s)

main  density=1.0  N=4  done  (207s)

main  density=1.0  N=5  done  (223s)

main  density=1.0  N=6  done  (233s)

main  density=1.0  N=7  done  (265s)

main  density=1.0  N=8  done  (290s)

scale N=9 done

scale N=10 done

channel LoS + 4 dB shadowing N=4 done

channel LoS + 4 dB shadowing N=6 done

channel LoS + 4 dB shadowing N=8 done

channel Mixed LoS/NLoS N=4 done

channel Mixed LoS/NLoS N=6 done

channel Mixed LoS/NLoS N=8 done

=== TABLE A: main results (mean ± 95% CI); gains/p-values are Proposed vs. method ===

N  density                            method  DR_mean  DR_ci95  EE_mean  EE_ci95  Jain_mean  gain_DR_%  gain_EE_%  p_DR_holm  p_EE_holm

3      0.5          Base SQP (Alnakhli 2024)  19.1538   0.6584   0.0534   0.0027     0.8015     0.0000     0.0000     0.0000        0.0

3      0.5               Equal MIS bandwidth  17.0739   0.4671   0.0481   0.0026     0.9042    12.1816    10.9532     0.0000        0.0

3      0.5 Proposed (SE-aware + alternating)  19.1538   0.6584   0.0534   0.0027     0.8015        NaN        NaN        NaN        NaN

3      0.5                  Random Selection  22.0995   1.5252   0.0172   0.0016     0.7599   -13.3293   210.2104     1.0000        0.0

3      0.8          Base SQP (Alnakhli 2024)  18.9189   0.6757   0.0523   0.0028     0.7838     0.0000     0.0000     0.0000        0.0

3      0.8               Equal MIS bandwidth  16.6758   0.4873   0.0465   0.0026     0.8951    13.4509    12.3198     0.0000        0.0

3      0.8 Proposed (SE-aware + alternating)  18.9189   0.6757   0.0523   0.0028     0.7838        NaN        NaN        NaN        NaN

3      0.8                  Random Selection  20.9460   1.3786   0.0178   0.0018     0.7694    -9.6780   194.2880     1.0000        0.0

3      1.0          Base SQP (Alnakhli 2024)  20.2574   0.5032   0.0478   0.0017     0.7076    -3.2754     8.1063     1.0000        0.0

3      1.0               Equal MIS bandwidth  17.2748   0.1782   0.0414   0.0016     0.8604    13.4248    24.7730     0.0000        0.0

3      1.0 Proposed (SE-aware + alternating)  19.5939   0.4309   0.0516   0.0017     0.7168        NaN        NaN        NaN        NaN

3      1.0                  Random Selection  22.4913   0.9876   0.0172   0.0013     0.6854   -12.8823   199.6775     1.0000        0.0

4      0.5          Base SQP (Alnakhli 2024)  17.4729   0.7608   0.0518   0.0025     0.7537    -1.0267     4.5432     1.0000        0.0

4      0.5               Equal MIS bandwidth  14.6830   0.5055   0.0438   0.0019     0.8698    17.7791    23.4603     0.0000        0.0

4      0.5 Proposed (SE-aware + alternating)  17.2935   0.7325   0.0541   0.0026     0.7416        NaN        NaN        NaN        NaN

4      0.5                  Random Selection  15.6333   1.0834   0.0102   0.0011     0.6900    10.6197   431.8986     0.0001        0.0

4      0.8          Base SQP (Alnakhli 2024)  19.9629   0.5824   0.0569   0.0021     0.6808     6.5846    10.3696     0.0000        0.0

4      0.8               Equal MIS bandwidth  16.7065   0.2247   0.0479   0.0014     0.8387    27.3604    31.1277     0.0000        0.0

4      0.8 Proposed (SE-aware + alternating)  21.2774   0.6067   0.0628   0.0023     0.5932        NaN        NaN        NaN        NaN

4      0.8                  Random Selection  12.5912   0.4322   0.0071   0.0004     0.6341    68.9861   789.6739     0.0000        0.0

4      1.0          Base SQP (Alnakhli 2024)  21.6037   0.4342   0.0600   0.0021     0.6590    12.8594    13.0123     0.0000        0.0

4      1.0               Equal MIS bandwidth  20.1337   0.0916   0.0561   0.0017     0.8360    21.0992    20.9731     0.0000        0.0

4      1.0 Proposed (SE-aware + alternating)  24.3818   0.5182   0.0678   0.0023     0.5332        NaN        NaN        NaN        NaN

4      1.0                  Random Selection  12.7681   0.3795   0.0071   0.0004     0.5985    90.9580   849.5313     0.0000        0.0

5      0.5          Base SQP (Alnakhli 2024)  15.2020   0.5318   0.0515   0.0018     0.6712     9.4246    11.9614     0.0000        0.0

5      0.5               Equal MIS bandwidth  13.6632   0.4350   0.0465   0.0016     0.7885    21.7489    24.0326     0.0000        0.0

5      0.5 Proposed (SE-aware + alternating)  16.6347   0.5725   0.0576   0.0020     0.5785        NaN        NaN        NaN        NaN

5      0.5                  Random Selection   9.6260   0.4266   0.0043   0.0003     0.6416    72.8107  1244.6263     0.0000        0.0

5      0.8          Base SQP (Alnakhli 2024)  15.1828   0.3583   0.0497   0.0015     0.5886    23.1212    31.9698     0.0000        0.0

5      0.8               Equal MIS bandwidth  14.0659   0.2101   0.0463   0.0013     0.7938    32.8976    41.7023     0.0000        0.0

5      0.8 Proposed (SE-aware + alternating)  18.6932   0.3575   0.0657   0.0016     0.3969        NaN        NaN        NaN        NaN

5      0.8                  Random Selection   9.0044   0.2446   0.0039   0.0002     0.5734   107.6013  1587.4013     0.0000        0.0

5      1.0          Base SQP (Alnakhli 2024)  15.3193   0.2822   0.0476   0.0014     0.5453    30.3481    45.1400     0.0000        0.0

5      1.0               Equal MIS bandwidth  14.2143   0.0428   0.0443   0.0011     0.8171    40.4815    55.9913     0.0000        0.0

5      1.0 Proposed (SE-aware + alternating)  19.9684   0.3158   0.0691   0.0013     0.3258        NaN        NaN        NaN        NaN

5      1.0                  Random Selection   8.8374   0.1752   0.0038   0.0002     0.5647   125.9546  1727.4506     0.0000        0.0

6      0.5          Base SQP (Alnakhli 2024)  15.1049   0.4424   0.0549   0.0017     0.5981     9.7666    12.7432     0.0000        0.0

6      0.5               Equal MIS bandwidth  12.0406   0.2543   0.0440   0.0012     0.8005    37.7020    40.5634     0.0000        0.0

6      0.5 Proposed (SE-aware + alternating)  16.5802   0.4251   0.0619   0.0016     0.4677        NaN        NaN        NaN        NaN

6      0.5                  Random Selection   6.8618   0.2496   0.0025   0.0002     0.5861   141.6296  2358.6940     0.0000        0.0

6      0.8          Base SQP (Alnakhli 2024)  17.1616   0.4141   0.0584   0.0018     0.4848    20.5806    22.0299     0.0000        0.0

6      0.8               Equal MIS bandwidth  13.8405   0.1500   0.0472   0.0011     0.8179    49.5147    51.1775     0.0000        0.0

6      0.8 Proposed (SE-aware + alternating)  20.6936   0.3317   0.0713   0.0019     0.3610        NaN        NaN        NaN        NaN

6      0.8                  Random Selection   6.7958   0.1346   0.0024   0.0001     0.5589   204.5077  2931.2410     0.0000        0.0

6      1.0          Base SQP (Alnakhli 2024)  16.9349   0.3333   0.0555   0.0017     0.4253    35.0388    34.7370     0.0000        0.0

6      1.0               Equal MIS bandwidth  16.4366   0.0401   0.0538   0.0011     0.8235    39.1323    39.0205     0.0000        0.0

6      1.0 Proposed (SE-aware + alternating)  22.8687   0.2768   0.0748   0.0016     0.2926        NaN        NaN        NaN        NaN

6      1.0                  Random Selection   6.7699   0.1077   0.0023   0.0001     0.5540   237.7989  3140.3274     0.0000        0.0

7      0.5          Base SQP (Alnakhli 2024)  13.4977   0.3751   0.0510   0.0016     0.5297    20.6468    22.3637     0.0000        0.0

7      0.5               Equal MIS bandwidth  11.9427   0.2583   0.0451   0.0012     0.7628    36.3555    38.3756     0.0000        0.0

7      0.5 Proposed (SE-aware + alternating)  16.2845   0.4162   0.0624   0.0017     0.4056        NaN        NaN        NaN        NaN

7      0.5                  Random Selection   5.5525   0.1582   0.0017   0.0001     0.5709   193.2849  3565.5992     0.0000        0.0

7      0.8          Base SQP (Alnakhli 2024)  13.9206   0.2788   0.0484   0.0013     0.3869    40.0407    47.7302     0.0000        0.0

7      0.8               Equal MIS bandwidth  13.3041   0.1267   0.0462   0.0009     0.7767    46.5308    54.6686     0.0000        0.0

7      0.8 Proposed (SE-aware + alternating)  19.4946   0.2549   0.0715   0.0013     0.2533        NaN        NaN        NaN        NaN

7      0.8                  Random Selection   5.5016   0.1003   0.0017   0.0001     0.5490   254.3443  4200.1960     0.0000        0.0

7      1.0          Base SQP (Alnakhli 2024)  14.0939   0.2954   0.0476   0.0014     0.3389    44.0384    57.6597     0.0000        0.0

7      1.0               Equal MIS bandwidth  13.4142   0.0296   0.0452   0.0009     0.8153    51.3374    65.7533     0.0000        0.0

7      1.0 Proposed (SE-aware + alternating)  20.3007   0.2341   0.0750   0.0010     0.2086        NaN        NaN        NaN        NaN

7      1.0                  Random Selection   5.4733   0.0677   0.0016   0.0001     0.5456   270.9059  4487.0303     0.0000        0.0

8      0.5          Base SQP (Alnakhli 2024)  14.4064   0.3662   0.0555   0.0016     0.4552    19.5183    21.4939     0.0000        0.0

8      0.5               Equal MIS bandwidth  11.6957   0.1849   0.0451   0.0010     0.7729    47.2182    49.4679     0.0000        0.0

8      0.5 Proposed (SE-aware + alternating)  17.2182   0.3378   0.0674   0.0015     0.3446        NaN        NaN        NaN        NaN

8      0.5                  Random Selection   4.6466   0.1162   0.0012   0.0001     0.5618   270.5588  5374.6303     0.0000        0.0

8      0.8          Base SQP (Alnakhli 2024)  15.5484   0.3403   0.0567   0.0014     0.3366    35.1483    35.7199     0.0000        0.0

8      0.8               Equal MIS bandwidth  12.9874   0.1017   0.0475   0.0008     0.8131    61.7983    62.2278     0.0000        0.0

8      0.8 Proposed (SE-aware + alternating)  21.0134   0.2738   0.0770   0.0014     0.2508        NaN        NaN        NaN        NaN

8      0.8                  Random Selection   4.6239   0.0766   0.0012   0.0000     0.5313   354.4500  6271.0859     0.0000        0.0

8      1.0          Base SQP (Alnakhli 2024)  15.4107   0.2933   0.0552   0.0014     0.2955    47.0926    47.0312     0.0000        0.0

8      1.0               Equal MIS bandwidth  15.0424   0.0301   0.0540   0.0010     0.8190    50.6939    50.4185     0.0000        0.0

8      1.0 Proposed (SE-aware + alternating)  22.6680   0.2155   0.0812   0.0014     0.1972        NaN        NaN        NaN        NaN

8      1.0                  Random Selection   4.5585   0.0519   0.0012   0.0000     0.5393   397.2687  6644.6743     0.0000        0.0

=== TABLE B: ablation ===

N  density                            method  DR_mean  DR_ci95  EE_mean  EE_ci95  Jain_mean  gain_DR_%  gain_EE_%  p_DR_holm  p_EE_holm

4      0.8           Base SLSQP random start  19.1230   0.4548   0.0546   0.0018     0.7422    11.2658    14.9948        0.0     0.0000

4      0.8          Base SQP (Alnakhli 2024)  19.9629   0.5824   0.0569   0.0021     0.6808     6.5846    10.3696        0.0     0.0000

4      0.8                     Base SQP-WoPC  24.1305   0.7307   0.0060   0.0002     0.6842   -11.8237   941.0082        1.0     0.0000

4      0.8 Proposed (SE-aware + alternating)  21.2774   0.6067   0.0628   0.0023     0.5932        NaN        NaN        NaN        NaN

4      0.8                     Proposed-WoPC  25.8870   0.7649   0.0065   0.0002     0.5966   -17.8065   870.3757        1.0     0.0000

4      0.8           Proposed: SE-aware only  21.3500   0.6054   0.0609   0.0022     0.5952    -0.3399     3.0711        1.0     0.0000

4      0.8        Proposed: alternating only  19.8913   0.5836   0.0587   0.0022     0.6793     6.9685     7.0317        0.0     0.0000

6      0.8           Base SLSQP random start  16.4027   0.2184   0.0559   0.0013     0.6981    26.1600    27.5965        0.0     0.0000

6      0.8          Base SQP (Alnakhli 2024)  17.1616   0.4141   0.0584   0.0018     0.4848    20.5806    22.0299        0.0     0.0000

6      0.8                     Base SQP-WoPC  20.6587   0.5104   0.0034   0.0001     0.4857     0.1688  1971.1772        1.0     0.0000

6      0.8 Proposed (SE-aware + alternating)  20.6936   0.3317   0.0713   0.0019     0.3610        NaN        NaN        NaN        NaN

6      0.8                     Proposed-WoPC  25.0691   0.4174   0.0042   0.0001     0.3608   -17.4539  1606.7938        1.0     0.0000

6      0.8           Proposed: SE-aware only  20.7241   0.3311   0.0707   0.0019     0.3619    -0.1471     0.8761        1.0     0.0000

6      0.8        Proposed: alternating only  17.1455   0.4135   0.0587   0.0018     0.4847    20.6937    21.4429        0.0     0.0000

8      0.8           Base SLSQP random start  15.0780   0.1451   0.0551   0.0010     0.7387    39.3645    39.7684        0.0     0.0000

8      0.8          Base SQP (Alnakhli 2024)  15.5484   0.3403   0.0567   0.0014     0.3366    35.1483    35.7199        0.0     0.0000

8      0.8                     Base SQP-WoPC  18.6825   0.4127   0.0023   0.0001     0.3374    12.4767  3196.6145        0.0     0.0000

8      0.8 Proposed (SE-aware + alternating)  21.0134   0.2738   0.0770   0.0014     0.2508        NaN        NaN        NaN        NaN

8      0.8                     Proposed-WoPC  25.4430   0.3426   0.0032   0.0000     0.2501   -17.4100  2320.6576        1.0     0.0000

8      0.8           Proposed: SE-aware only  21.0486   0.2743   0.0768   0.0014     0.2509    -0.1673     0.2093        1.0     0.0041

8      0.8        Proposed: alternating only  15.5295   0.3402   0.0568   0.0014     0.3369    35.3131    35.6504        0.0     0.0000

=== TABLE C: scalability and channel robustness ===

exp              channel  N  density                            method  DR_mean  DR_ci95  EE_mean  EE_ci95  Jain_mean  gain_DR_%  gain_EE_%  p_DR_holm  p_EE_holm

channel LoS + 4 dB shadowing  4      0.8          Base SQP (Alnakhli 2024)  28.2112   1.8445   0.0763   0.0075     0.5375     7.7791    19.5119        0.0        0.0

channel LoS + 4 dB shadowing  4      0.8 Proposed (SE-aware + alternating)  30.4058   1.9543   0.0912   0.0086     0.4708        NaN        NaN        NaN        NaN

channel LoS + 4 dB shadowing  6      0.8          Base SQP (Alnakhli 2024)  22.3638   0.8767   0.0800   0.0050     0.3708    39.6037    46.6312        0.0        0.0

channel LoS + 4 dB shadowing  6      0.8 Proposed (SE-aware + alternating)  31.2207   1.2982   0.1174   0.0087     0.2517        NaN        NaN        NaN        NaN

channel LoS + 4 dB shadowing  8      0.8          Base SQP (Alnakhli 2024)  20.4836   0.9777   0.0749   0.0047     0.2794    67.2189    72.9945        0.0        0.0

channel LoS + 4 dB shadowing  8      0.8 Proposed (SE-aware + alternating)  34.2525   1.2972   0.1297   0.0077     0.1691        NaN        NaN        NaN        NaN

channel       Mixed LoS/NLoS  4      0.8          Base SQP (Alnakhli 2024)  48.7151   3.6089   0.0592   0.0074     0.3923     6.4542    16.0616        0.0        0.0

channel       Mixed LoS/NLoS  4      0.8 Proposed (SE-aware + alternating)  51.8593   3.8455   0.0687   0.0079     0.3725        NaN        NaN        NaN        NaN

channel       Mixed LoS/NLoS  6      0.8          Base SQP (Alnakhli 2024)  36.3683   2.4854   0.0635   0.0071     0.2688    43.6628    57.8205        0.0        0.0

channel       Mixed LoS/NLoS  6      0.8 Proposed (SE-aware + alternating)  52.2477   3.5483   0.1002   0.0117     0.2164        NaN        NaN        NaN        NaN

channel       Mixed LoS/NLoS  8      0.8          Base SQP (Alnakhli 2024)  32.0630   2.0329   0.0604   0.0061     0.1948    85.3063    98.0017        0.0        0.0

channel       Mixed LoS/NLoS  8      0.8 Proposed (SE-aware + alternating)  59.4147   2.9439   0.1196   0.0105     0.1413        NaN        NaN        NaN        NaN

scale           LoS (base)  9      0.5          Base SQP (Alnakhli 2024)  13.0168   0.5337   0.0518   0.0024     0.3997    34.8074    36.3896        0.0        0.0

scale           LoS (base)  9      0.5 Proposed (SE-aware + alternating)  17.5476   0.6761   0.0706   0.0029     0.2894        NaN        NaN        NaN        NaN

scale           LoS (base) 10      0.5          Base SQP (Alnakhli 2024)  14.2331   0.5774   0.0567   0.0027     0.3630    28.9490    30.1536        0.0        0.0

scale           LoS (base) 10      0.5 Proposed (SE-aware + alternating)  18.3534   0.5727   0.0738   0.0027     0.2755        NaN        NaN        NaN        NaN

=== TABLE D: runtime (ms per network) ===

N   method

3   Base SQP (Alnakhli 2024)              3.25

Proposed (SE-aware + alternating)    13.87

4   Base SQP (Alnakhli 2024)              3.13

Proposed (SE-aware + alternating)    17.04

5   Base SQP (Alnakhli 2024)              2.44

Proposed (SE-aware + alternating)    18.97

6   Base SQP (Alnakhli 2024)              2.75

Proposed (SE-aware + alternating)    12.95

7   Base SQP (Alnakhli 2024)              4.12

Proposed (SE-aware + alternating)    33.39

8   Base SQP (Alnakhli 2024)              5.33

Proposed (SE-aware + alternating)    23.86

9   Base SQP (Alnakhli 2024)              4.58

Proposed (SE-aware + alternating)    27.21

10  Base SQP (Alnakhli 2024)              5.29

Proposed (SE-aware + alternating)    32.36

=== TABLE E: QoS factor beta sweep (N=6, density 0.8) ===

DR      EE    Jain

beta method

0.5  Base SQP (Alnakhli 2024)           11.3993  0.1531  0.4807

Proposed (SE-aware + alternating)  13.6079  0.1849  0.3570

0.6  Base SQP (Alnakhli 2024)           13.4019  0.1204  0.4817

Proposed (SE-aware + alternating)  16.0468  0.1460  0.3573

0.7  Base SQP (Alnakhli 2024)           15.3288  0.0892  0.4825

Proposed (SE-aware + alternating)  18.4158  0.1085  0.3574

0.8  Base SQP (Alnakhli 2024)           17.1844  0.0593  0.4832

Proposed (SE-aware + alternating)  20.7122  0.0724  0.3574

0.9  Base SQP (Alnakhli 2024)           18.9729  0.0308  0.4838

Proposed (SE-aware + alternating)  22.9412  0.0377  0.3573

Total time 438 s